Errata
FEMA 350

July 28, 2026

| Section | Page | Correction |
| --- | --- | --- |
| 2.13.1 | 2-26 | Reference to Section 3.3.2.4 should instead be to Section 3.3.2.5. |
| 3.3.2.5 | 3-16 | Reference to “20ft-lb. at –20oF” in first line should instead be to “20ft-lb. at 0oF” |
| 3.5.1 | 3-27 | Table 3-2, Beam/Column Relations: 1st line should read as follows: Panel Zone Strength No Requirement (OMF) |
| 3.5.2 | 3-30 | Table 3-3. Beam/Column Relations: 2nd line should read as follows: Column/beam bending strength: SMF: Section 2.9.1 |
| 3.5.4 | 3-35 | Table 3-5. Beam/Column/Flange Plate (FP) Relations, should read as follows: Panel Zone Strength SMF: Section 3.3.3.2 Column/beam bending strength ratio SMF: Section 2.9.1 |
| 3.5.4.1 | 3-38 | Step 6 should read: Determine the required panel zone thickness according to the methods of Section 3.3.3.2. For purposes of this calculation, substitute db+(tplt+tplb) for db and the quantity db+tplt+tplb2for db - tfb |
| 3.5.5 | 3-40 | Table 3-6. Beam/Column Relations: 2nd line should read as follows: Column/beam bending strength: SMF: Section 2.9.1 |
| 3.5.5.1 | 3-41 | Step 2, Item d, should read: If Mf<RyZbFy the design is acceptable. If Mf is greater than the limit, increase c. The value of c should not exceed 0.25bf. |
| 3.6.1 | 3-44 | Table 3-8. Beam/Column Relations: 1st and 2nd lines should read: Panel zone strength: SMF: Sec. 3.3.3.2, Sec. 3.6.1.1 Step 9 Column/beam bending strength ratio: SMF: Sec. 2.9.1 |
| 3.6.1.1 | 3-45 | In Step 1, the reference to Section 3.2.7 should instead be to Section 3.2.6. Step 2 should read: - Select end plate bolt size by solving Equation 3-18 for Tub and selecting bolt type and Abolt as required: (3-18) where: Tub = 90Abolt for A325 bolts = 113Abolt for A490 bolts and do and di are as defined in Figure 3-14 |
| 3.6.1.1 | 3-47 | Equation 3-29 should read: s=C1C2C2+2C12bfc−4k1 and sentence beneath Equation 3-29 should read: If tfc is less than the calculated value, a column with a thicker flange must be selected. |

| Section | Page | Correction |
| --- | --- | --- |
| 3.6.1 | 3-43 | Figure 3-13 – Replace with: |

| Section | Page | Correction |
| --- | --- | --- |
| 3.6.1.1 | 3-48 | Figure 3-14 should appear as: |
| 3.6.2 | 3-50 | Table 3-9. Critical Beam Parameters. Flange Thickness up to 1” |
| 3.6.2 | 3-50 | Table 3-9. Beam/Column Relations. 1st and 2nd lines should read: Panel zone strength: SMF Sec. 3.6.2.1 Step 8. Column/beam bending strength ratio: SMF: Sec. 2.9.1 |

| Section | Page | Correction |
| --- | --- | --- |
| 3.6.2 | 3-53 | Figure 3-16 – Replace with: |

| Section | Page | Correction |
| --- | --- | --- |
| 3.6.2 | 3-49 | Figure 3-15 – Replace with: |
| 3.6.2.1.1 | 3-51 | Step 2, reference to Equation 3-32 should be to Equation 3-31. Sentence below Equation 3-32 should read: Where Tb is the minimum bolt pretension per Table J3.1 of AISC-LRFD and Ffu is as defined in Equation 3-36. |
| 3.6.2.1.1 | 3-52 | Step 6 should read “Check column web ...” |
| 3.6.3 | 3-55 | Table 3-10. Critical Beam Column Relations: 1st and 2nd lines should read: Panel zone strength: SMF: Sec. 3.6.3.1, Step 3 Column/beam bending strength ratio SMF: Sec. 2.9.1 |
| 3.6.3.1 | 3-58 | Equation 3-47 should read: Mfail=Fu−bZb−2dbt+0.062tfbdb−tfbLTF3 |
| 3.7.1 | 3-62 | Table 3-12. Critical Beam Column Relations. 1st and 2nd lines should read: Panel zone strength: SMF: Sec. 3.7.1.2, Step 3 Column/beam bending strength ratio SMF: Sec. 2.9.1 |
| 3.7.1.2 | 3-64 | Figure 3-21 should appear as: |
| 3.7.1.2 | 3-66 | Step 6: reference to Figure 22 should be to Figure 3-22. |
| 3.7.1.2 | 3-66 | Equation 3-61 should read: Mfail=2a'−dbt4wFyTtft2db−tstem4a'b'−dbTa'+b' |
| 3.7.1.2 | 3-66 | Equation 3-64 should read: Mfail=Ntbdb+tstemTub+wFyttft216 {a'a'a'+b' |
| 3.7.1.2 | 3-67 | Equation 3-65 should read: Mfail=FubmZb−2dbt+0.062tfbdb−tfbLTF3 |

| Section | Page | Correction |
| --- | --- | --- |
| Appendix A | A-6 | Equation A-3 should read: λ=e−βUTKx−k2bβUT |